# CSS 422: Hardware and Computer Organization

# Homework: Sequential Circuits

# Instructor Rob Nash

## Student Name: Thuan Tran

A full-adder is a combinational circuit (memory-less) that forms the arithmetic sum of two input bits (say a and b) and a carry in (Cin,  so three input bits total).  The full-adder provides two outputs in the form of the (S)um and the carry out (Cout).  The input bits a and b represent the terms to be added, but the full-adder needs to also consider the carry in bit, too.  The following questions are each worth 1 point.

- Construct a truth table for the Full-Adder

| INPUT |  |  | OUTPUT |  |
| --- | --- | --- | --- | --- |
| A | B | Cin | Sum | Cout |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

- Construct a K-Map for the sum output (S) and derive the most reduced Boolean equation from your map.

|  | !A!B | !AB | AB | A!B |
| --- | --- | --- | --- | --- |
| !Cin |  | 1 |  | 1 |
| Cin | 1 |  | 1 |  |

Derived equation: !A!BC + !AB!C + ABC + A!B!C

- Construct a K-Map for the carry out (Cout) and derive the most reduced Boolean equation from your map.

|  | !A!B | !AB | AB | A!B |
| --- | --- | --- | --- | --- |
| !Cin |  |  | 1 |  |
| Cin |  | 1 | 1 | 1 |

Derived equation: AB + BC + AC

- Using Boolean Algebra, prove that S can be expressed as the XOR of the three inputs.  Said another way, show the steps in Boolean Algebra that prove:

S = a XOR b XOR c

= (A XOR B) XOR C

=  (A!B + !AB) XOR C

= (A!B + !AB) !C + C !(A!B + !AB)

= A!B!C + !AB!C + C( (!A + B) ( A + !B) )

Now focus on this part: C( (!A + B) ( A + !B) )

= C ( !AA + BA + !A!B + !BB)

= ABC + !A!BC ( Get rid of !AA and !BB)

Now plug back the result we have

A!B!C + !AB!C + ABC + A!BC, which are the same as answer to question 2

- Using algebraic manipulation (like in the previous problem), prove that:

Cout = ab + (a XOR b)*Cin

= AB + (!AB + A!B)C

= (AB + (!AB + A!B) ) (AB + C)

= ( AB + AB + !AB + A!B) (AB + C)

= ( A(B+!B) + B(A + !A) ) (AB + C)

= (A + B) (AB + C)

= AAB + AC + BAB + BC

=AB + AC + AB +BC = AB + AC +BC

- Based on the previous formulas from (4) and (5), draw the circuit for the full-adder in Logisim; copy and paste a screen shot here and also submit the “.circ” file to the website.  Note that just a screenshot is not sufficient.

- Test your circuit and describe both the circuit and your test procedure.

  - Does this adder look like the 1-bit adder you produced in the previous homework?

    - If yes or no, why?

Yes, this looks like the 1bit adder. That is if we trace along the path to the Sum output, It can only be 1 if only 1 of the input to the second XOR gate is 1. Meaning only Cin can be and other must be 0 . Or Cin is 0 and at most 1 of A and B be 1. This correspond to the truth table

Test procedure: Manually adjust the input and record the output

Quick way: Analyze circuit and truth table was generated

| Xin | Yin | Cin | S | Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

- (3 points) The following sequential circuit includes state and a full-adder.  Inputs are a,b,Cin and outputs are the next state of S and Cout  Complete the following truth table to describe the sequential circuit presented here.  (S is the output Sum, the D-ff stores the resultant carry-out Cout for use as the next carry-in, available on Q)

Hints: What is the relationship between the Carry-out before the clock and the Carry-in after the clock?

Second Hint: Do the columns “Before Clock” first.

| Xin | Yin | Carry-in (or Q) Before Clock | S Before Clock | Carry-out Before Clock | S After Clock | Carry-out After Clock |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |

- (5 pts) A sequential circuit has two D-FFs (A, B) and two inputs x and y, with one output z.  The flip-flop input equations and circuit output is defined as:

Da = ~xy + xA

Db = ~xB + xA

z = B

Draw the logic diagram of this circuit and test it in Logisim.  Copy and paste a screenshot here and attach the “.circ” file separately.

- (2 pts) Construct a state diagram (a visual representation of an FSM) of this circuit.

| Current |  | Input |  | Next |  | Output |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | X | Y | A | B | Z |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |

- (5 pts) Design a state-based circuit that uses 3 bits to count down from 111 to 000.  This 3-bit count-down counter will use 3 JK flip-flops and one input x.  If x is 0, don’t change state.  If x==1, then progress the state sequence from 1112 to 0002.  Start by drawing a state diagram (a visual representation of an FSM) here.

- Complete the following excitation table to determine the inputs Ja, Ka, Jb, Kb, Jc, and Kc respectively for your previous count-down counter.  Do note the unusual counting order is intentional.

| A | B | C | x | A t+1 | B t+1 | C t+1 | Ja | Ka | Jb | Kb | Jc | Kc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | x | 0 | x | 0 | X |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | x | 1 | X | 0 | X |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | x | 0 | X | x | 0 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | X | 1 | x | x | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | X | X | 0 | 0 | X |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | X | X | 1 | 0 | X |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | X | X | 0 | X | 0 |
| 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | X | X | 1 | X | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | X | 0 | 0 | X | 0 | X |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 | X | 0 | 1 | X | 0 | X |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | X | 0 | 0 | X | X | 0 |
| 1 | 0 | 1 | 1 | 1 | 1 | 1 | X | 0 | 1 | X | X | 0 |
| 1 | 1 | 0 | 0 | 1 | 1 | 0 | X | 0 | X | 0 | 0 | X |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | X | 1 | X | 1 | 1 | X |
| 1 | 1 | 1 | 0 | 1 | 1 | 1 | X | 0 | X | 0 | X | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | X | 1 | X | 1 | X | 1 |

- Draw K-maps and then derive Boolean equations using those K-maps.  Reduce the equation to its simplest form.

Ja = 1 big 8 loop and a big 4 loop

= A + Bx

|  | !A!B | !AB | AB | A!B |
| --- | --- | --- | --- | --- |
| !C!X |  |  | X | X |
| !CX |  | 1 | X | X |
| CX |  | 1 | X | X |
| C!X |  |  | X | X |

Ka = !A + Bx

|  | !A!B | !AB | AB | A!B |
| --- | --- | --- | --- | --- |
| !C!X | X | X |  |  |
| !CX | X | X | 1 |  |
| CX | X | X | 1 |  |
| C!X | X | X |  |  |

Jb = B + X

|  | !A!B | !AB | AB | A!B |
| --- | --- | --- | --- | --- |
| !C!X |  | X | X |  |
| !CX | 1 | X | X | 1 |
| CX | 1 | X | X | 1 |
| C!X |  | X | X |  |

Kb = !B + X

|  | !A!B | !AB | AB | A!B |
| --- | --- | --- | --- | --- |
| !C!X | X |  |  | X |
| !CX | X | 1 | 1 | X |
| CX | X | 1 | 1 | X |
| C!X | X |  |  | X |

Jc = C + ABx

|  | !A!B | !AB | AB | A!B |
| --- | --- | --- | --- | --- |
| !C!X |  |  |  |  |
| !CX |  |  | 1 |  |
| CX | X | X | X | X |
| C!X | X | X | X | X |

Kc = !C + ABx

|  | !A!B | !AB | AB | A!B |
| --- | --- | --- | --- | --- |
| !C!X | X | X | X | X |
| !CX | X | X | X | X |
| CX |  |  | 1 |  |
| C!X |  |  |  |  |

- Draw this circuit in Logisim and include a screen shot of the circuit here, as well as attaching the “.circ” file for analysis.

- Test your system and report on your testing.

  - What type of testing did you do?

    - What did it reveal?

I set the clock to auto tick and log the value. And also manually “poke” the input to compare the result

I notice that this acts like a counter up from 000 to 111. I only need to switch the position of the table from ABC to CAB